MEGARA Control System.

Fiber MOS Assignment Tool (FMAT).

User Manual.

GTC Code:

Code:		TEC/MEG/174

Issue:		2.B

Date:		21/1/2016

Pages:		46

| Authors: | Pedro Gómez-Cambronero Alvarez María Luisa García Vargas Emma Mujica Alvarez |
| --- | --- |
| Revised by: | Ana Pérez Calpena África Castillo Morales Jorge Iglesias Páramo Armando Gil de Paz |
| Approved by: | Armando Gil de Paz María Luisa García Vargas |

Distribution List:

| Name | Affiliation | Date |
| --- | --- | --- |
| GRANTECAN | GRANTECAN | 21/1/2016 |
| MEGARA Team | All Partners | 21/1/2016 |

Acronyms:

ASCII			American Standard Code for Information Interchange

CB			Configuration Block

CP			Configuration Plan

FC			Folded-Cassegrain

FMAT			Fiber MOS Assignment Tool

FMPT			Fiber MOS Positioning Tool

FOV			Field of View

GCS 			GTC Control System

GFCCS			GTC Folded-Cassegrain Coordinate System

GTC			Gran Telescopio Canarias

IAA			Instituto de Astrofísica de Andalucía (CSIC)

IFU			Integral Field Unit

LCB			Large Compact Bundle

MEGARA		Multi Espectrógrafo en GTC de Alta Resolución para Astronomía

MCS			MEGARA Control System MEGARA

MOPSS		MEGARA Observing Preparation Software Suite

MOS			Multi-Object Spectroscopy

OB			Observing Block

RP			Robotic Positioner

SP			Sky Point

S/W			Software

PA			 Position Angle

PP			Projection Point

TBC 			To Be Confirmed

TBD 			To Be Done

TBU			To Be Updated

Change Control

| Issue | Date | Section | Page | Change description |
| --- | --- | --- | --- | --- |
| 1.A | 26/01/15 | All | All | New Issue |
| 1.B | 23/06/15 | 3 3.2.1 4.2.1 5.2.1 5.2.5 5.2.7 5.3 5.6 5.8 5.9 6 | 15 17 23 28 32 33 35 41 43 44 43 | Add new functionality: automatic assignment Change the ID of the source type REFERENCESOURCE to REFERENCE. Changes the fmat.props information. Add new menu tabs. Add new filters to the query panel Block panel: the possibility to display or not the blocks in the Sky panel. Add new block at a specific sky position by filling the central coordinates and PA. New section: How to assign and de-assign sources automatically to a block. New section: How to clone a block. New section: How to save the blocks. New section: Future Developments |
| 2.A | 30/08/15 | 2.1.4 4.2 4.2.1 5 5.2.1 6 2.1.2 All | 14 21 24 27 28 43 11,12 All | Changed the Fiber MOS layout in the FMAT Add new properties file fmatFileChooser.dat Changes the fmat.props information. New MOS layout New Menu: Edit (undo/redo) New Help submenu: Manual Removed section “Future developments” Added new section explaining the geometry transformations in the MEGARA focal plane Style revision and introducing clarifications where necessary |
| 2.B | 03/12/15 | 3.2 | 16 | Only File->Load and File->save are now available |
|  |  | 4.2.2 | 25 | Remote properties implementation |
|  |  | 4.2.1 | 23 | Add description of megara.fmat.strict parameter |
|  |  | 5.10 | 43 | Describe strict implication at generation of configuration plan with issues |

Reference Documents

| Nº | Document Name | Code |
| --- | --- | --- |

Reference Documents (GTC codes)

| Nº | Document Name | Code |
| --- | --- | --- |

INDEX

1.	SUMMARY AND FMAT SCOPE	9

2.	MEGARA FIBER MOS INTRODUCTION	9

2.1	The Fiber MOS Geometry: Coordinate systems and MOS view on FMAT	11

2.1.1	Fiber MOS focal frame coordinate system	11

2.1.2	Coordinate system transformation	12

2.1.3	Fiber MOS “Hexmap” coordinate system	13

2.1.4	Fiber MOS layout in the FMAT	14

2.2	MEGARA focal plane cover	15

3.	FMAT Assignment Tool Overview	16

3.1	Configuration Block and Configuration Plan concepts	17

3.1.1	Configuration Plan (CP)	17

3.1.2	Configuration Block (CB)	17

3.2	The interface files: input and output files description	17

3.2.1	Input File for FMAT	18

3.2.2	Configuration Block file set from the FMAT: The FMPT input	20

4.	INSTALLING / execution	21

4.1	Software requirements	21

4.2	Local Directory structure	21

4.2.1	The fmat.props configuration file	24

4.2.2	The remote properties service	26

5.	USING THE FMAT	27

5.1	Starting	27

5.2	FMAT Graphic User Interface Description	27

5.2.1	Main FMAT Menu	28

5.2.2	The Schedule panel	31

5.2.3	The Sky panel	31

5.2.4	The Tree View panel	32

5.2.5	The Query panel	32

5.2.6	The Sources panel	33

5.2.7	The Blocks panel	33

5.3	How to create a new block	35

5.4	How to remove a block	37

5.5	How to assign and de-assign sources to the RPs	38

5.6	How to assign and de-assign sources automatically to a block	41

5.7	Choice between two possible RPs	42

5.8	How to clone a block	43

5.9	Save the blocks	44

5.10	Finishing the work: How to save a Configuration Plan	44

# SUMMARY AND FMAT SCOPE

This document contains the User Manual of the MEGARA Fiber MOS Assignment Tool (FMAT). The manual will allow the user to install the FMAT in his computer, to understand the FMAT functionality and to learn how to use the tool step by step in all its modes.

The FMAT will allow the user to prepare a Configuration Plan (CP) composed for several Configuration Blocks (CBs) for the MEGARA MOS mode by assisting the user in the assignment of their sources to the different in-use robotic positioners of the MEGARA MOS (up to 92 positioners). The current release supports both manual and automatic assignment through a user-friendly GUI.

The input for FMAT is a text/ASCII file with the information of the sky sources to be observed with the MEGARA MOS mode. This file can be the user catalog (without any positioner information) if the astronomer uses FMAT for the first time on a specific catalog or project. The input file can also be a preliminary output file generated previously by the user, in which part of the assignment work had been previously done but was not completed or the resulting Configuration Blocks were not validated. The description of the FMAT input file is given in section 3.2.1.

The final output of the FMAT is a set of files (one per Configuration Block). Each (text/ASCII) file contains the information of a set of sources that can be observed simultaneously in a single exposure with the MEGARA MOS and therefore is characterized by a common telescope pointing: Right Ascension (J2000.0), Declination (J2000.0) and Position Angle (PA). The telescope is supposed to point at the center of the MEGARA FOV (at the optical axis of the Folded-Cassegrain station) to these sky coordinates and then rotate the FC rotator to the PA of each CB to allow the requested orientation of the MEGARA FOV. Each file containing the information of a particular Configuration Block will be the input for the Fiber MOS Positioning Tool (FMPT). The description of the FMAT output file is given in section 3.2.2.

The calculation for the creation of the positioning/de-positioning files is out of the scope of the FMAT and is included as part of the FMPT functionality and documentation.

# MEGARA FIBER MOS INTRODUCTION

The MEGARA Instrument is composed of two modes, an Integral Field Unit (IFU) mode and a multi-object spectrograph (MOS) mode, that correspond to the two fiber bundles available: the Large Compact Bundle (LCB) and the Dispersed Bundle, or simply MOS bundle (see Figure 1).

The fibers that constitute the Large Compact Bundle (LCB) (100 m in core size) are arranged on a square area of 12.5 x 11.3 arcsec2 near the optical axis of the instrument. The fibers belonging to the dispersed bundle (MOS mode) (also 100 m in core diameter) can be positioned in the central 3.5 arcmin x 3.5 arcmin around the LCB IFU bundle by means of positioner robots (also referred here as Robotic Positioners or RPs).

Note that a total of 8 robotic positioners (orange hexagons in Figure 1) located at the edge of the instrument FOV are kept fixed in order to be used for measuring the sky background simultaneously with the observations with the LCB. The layout of the hexagonally-packed (and shaped) microlens array of the LCB IFU is also shown.

Figure 1 : Layout of the Large Compact (LCB) and Dispersed fiber bundles (MOS mode) of MEGARA.

The MEGARA MOS Mode (Dispersed bundle) is composed by a number of dispersed individual sets/mini-bundles of fibers (7 fibers each) that could be positioned within the central 3.5 arcmin x 3.5 arcmin around the LCB bundle thanks to the use of the RPs. Each RP has the functionality of positioning the 7-fiber minibundle anywhere within the corresponding positioner patrol area. The reference position for each positioner is the center of the central fiber of the 7-fiber mini-bundle.

These fibers go all together to a pseudo-slit, different from the one containing the LCB fibers. The current detailed-design configuration includes a total of 100 positioners, 92 of them dedicated to MOS observations, which yields a total of 644 fibers in the MOS pseudo-slit. The rest of positioners (8) would be evenly distributed along the pseudo-slit of the LCB IFU. Note that these 8 positioners, since are devoted to measuring the sky background, are not required to move, so they will be always kept in parking position. In Figure 2 we show the 3D view of the positioners system with the two original IFU bundles in the center (only the LCB bundle will be finally included in MEGARA).

Figure 2: 3D view of the Fiber MOS system. The LCB frame is identified and referred to drawing IN-MG-RA-100. All positioners are identical.

## The Fiber MOS Geometry: Coordinate systems and MOS view on FMAT

### Fiber MOS focal frame coordinate system

The Fiber MOS focal frame coordinate system is defined as follows (see Figure 3).

- The origin is in the center of the frame, coincident with the center of the FC FOV.

- The Z-axis runs in direction of the Z-axis of the GFCCSS against the incoming light.

- The Y-axis runs in direction of the Y-axis of the GFCCS, positive sense of Y-axis going away from the base plate.

- The X-axis forms a right-handed system with the two previous.

The Fiber MOS focal frame shall be used to define the position of the each positioner and, therefore, locate the origin of each individual Fiber MOS positioner coordinate system.

Figure 3: Fiber MOS Focal Frame coordinate system.

This coordinate system is the one called S0 coordinate system in the FMPT and is the one in which the FMAT generates the x, y coordinates for the final positions where the RP has to go in a given CB.

### Coordinate system transformation

The following formulae give the transformation between the cartesian (x,y) coordinates of the focal plane and the angles (α,β) as seen in Equation 1 and Figure 4.

Equation 1: Cartesian and polar transformations.

Figure 4: MEGARA cartesian and polar coordinates.

### Fiber MOS “Hexmap” coordinate system

The MEGARA focal plane arrangement is a Hexagonal pattern, which makes convenient to define a discrete Hexagonal reference system, with two axes at 120 degrees (see Figure 5); an arbitrary cell near the center is taken as [i=0,j=0].

The cartesian position of a hexmap index [i,j] is calculated by:

x = d * (i - (j + 1) / 2)

y = d * sqrt(3) * j / 2

Being d the distance between the center of the positioners.

This coordinate system makes easy to determine the neighbors of a given positioner [i,j]:

[i, j+1],[i+1, j+1],[i+1,j],[i, j-1],[i-1, j-1],[i-1, j]

This information is encoded in the configuration file fmatPositionerIds.dat1 where the correspondence between the positioner Ids and HexIds is performed.

Figure 5: Fiber MOS Hexmap coordinate system. The hex indexes are in blue.

### Fiber MOS layout in the FMAT

FMAT displays on the Sky Panel, for each CB, the MEGARA MOS FOV centered in the equatorial coordinates defined for the CB and oriented with the PA defined for that CB. The corresponding layout shows the MEGARA RPs as follows:

- RPs that are devoted to measure the sky background and hence are not mounted on the MOS pseudo-slit (or, simply, do not belong to the MOS), but on the LCB pseudo-slit instead, are painted in orange (the patrol areas).

- RPs that belong to the MOS but are disabled due to any reason (e.g., a mechanical failure, RP to be repaired, maintenance task, etc.) are painted in red (the patrol areas). The list of the disabled RPs is found in the configuration parameter megara.fmat.out_of_order_positioners within   the file fmat.props) included in the FMAT distribution. Once the instrument is in operation, we assume that the information regarding those RPs that are enabled will be available to the users at the time of preparing their proposals.

- All others RPs are in the same color as the background. The patrol areas are painted as grey open circles. The positioner headers are painted in green and the margins in orange. The RP identifications, same as shown in MEGARA drawings, are displayed on each RP area (see Figure 6).

Figure 6: MOS layout at FMAT. The layout shows the patrol areas and identification (number) of each positioner as well as the positioners (in orange) mounted on the LCB pseudo-slit (not usable in MOS mode) and the potential out-of-service positioners (in red, if any).

The green rectangle in the center of the FOV is used for moving and rotating the MEGARA focal plane with respect to the sky (in RA and Dec). By clicking and dragging on the rectangle we can move the FOV with the mouse while for rotating the FOV the mouse wheel should be used (the keyboard mapping of this function is TBD) or equivalent gestures in Mac OS X.

## MEGARA focal plane cover

The Fiber MOS is equipped with a cover that allows blocking one of the two halves of the focal plane (-X and +X in the focal plane reference frame); thus the optical path towards the microlens of some RPs could be blocked with the aim to reduce cross-talk between the light from fiber spectra that are adjacent on the MOS pseudo-slit and, therefore, on the detector.

The focal plane cover is composed of two plates that can be laterally moved each one independently from the other, in the plane just above the microlens, that is, very close to the focal plane. Each plate covers the corresponding half of the Fiber MOS. The positions for the different mini-bundles in the MOS pseudo-slit are assigned alternatively to the RPs of both sides of the Fiber MOS, so by covering one side of the Fiber MOS the distance between the projection of the spectra of two mini-bundles becomes as large as the projection of 7 fibers on the CCD. This allows observing (and reducing their spectra) sources covering a wide dynamic range in flux with a very small crosstalk in almost all cases.

Those RPs located in the border of the two subsets of RPs of the Fiber MOS will have their observing domain reduced by 50%. Half of these RPs will be assigned to one subset and the other half to the other.

The focal plane cover position shall be taken into account in the generation of the CPs, so that the projection points blocked by the cover shall not be assigned.

For an example of using the cover plate selection functionality, see 5.2.5 and Figure 15.

# FMAT Assignment Tool Overview

In summary, the FMAT allows the user to:

- Prepare each Configuration Block (that forms part of a Configuration Plan) for the MEGARA MOS. Each CB would contain the information of which sources have been assigned to which RPs and also the needed input information for the FMPT in order for this to calculate the positioning/de-positioning programs of the RPs.

- Prepare and validate a Configuration Plan with the MOS mode.

The FMAT is interactive through a user-friendly GUI that assists the user in the process of assigning the sources to the enable RPs. In particular, some examples of the main functionalities are:

- To prepare the CPs and CBs.

- To create, save and delete CBs.

- To show a graphical display of the Sky and the MEGARA focal plane with the MOS footprint on it. The MOS layout for each CB identifies the RPs and their patrol areas. Also, the RPs that either do not belong to the MOS mode or are not enabled at the moment of preparing the CBs are painted in different colors.

- To load user catalogs with astronomical sources to start the creation of CBs.

- To allow different filters for querying the user’s catalog with several criteria (priority range, magnitude range, label in the comment field etc.). The selected sources as result of a modification in these filters are simultaneously updated in all views (sky panel and source panel).

- To display automatically the sky area when selecting a given CB.

- To assign/de-assign sources to positioners and give a graphical view of the assignment.

- To allow the user to center and rotate the MEGARA FOV and assist the user by showing which assignments would have problems after moving the FOV and/or the PA.

- To resolve conflicts when different RPs can access a source.

- To allow the user automatically assign/de-assign sources to the CBs.

## Configuration Block and Configuration Plan concepts

### Configuration Plan (CP)

A Configuration Plan (CP) contains a set of MOS observations that the user prepares to submit via the GTC Phase-2 tool. The CP is a data structure consisting in a series of Configuration Blocks (CBs). Each CB will be used to define one (or more) OB in the Phase-2 tool with the MOS mode. The OB needs also the spectrograph information to complete this configuration. The provision of the spectrograph configuration information is out of the scope of the FMAT.

### Configuration Block (CB)

A CB is a data structure containing the information required to configure the Fiber MOS in a secure and efficient way, during the execution of an OB in the MEGARA MOS mode. A series of CBs constitutes a configuration plan (CP). The CP must be generated by means of the FMAT.

The CB must contain information to:

- Assign the user catalogue’s sources to the MOS positioners.

- Configure the Fiber MOS during the execution of the corresponding OB in the MEGARA MOS mode.

- Validate it given a set of predefined criteria: both general to the instrument (no collisions) and specific (number of sources assigned, combined priority, etc.).

In addition, each CB has to have an associated file with the information for positioning the robots in the assigned position and in the same file the information to allow the MEGARA Control System to bring the positioners back to the parking positions without any damage. The functionality of creating this file for positioning/de-positioning the robots is part of the scope of the FMPT.

## The interface files: input and output files description

There are two types of files of similar formats: the input file, which is the format used by the tool for loading and saving FMAT sessions, i.e. using the File->Load and File->Save, and the output file format (which contains information to be consumed by the FMPT) by using the option File->Generate configuration Plan.

### Input File for FMAT

The purpose of the FMAT I/O file, normally with extension .maf, is to store the current state of the assignment process. It comprises two sections: The configuration blocks (CBs) and the sources:

The file starts with a comment row (ignored by the S/W) with the column description of the CB:

# Id| Ra| Dec| Pos

These field names correspond to the Identification number (Id) for the Block, Right Ascension and Declination where the center of the MEGARA MOS FOV (which coincides with the FC optical axis) is pointing along the FC rotator Position Angle (between 0º and 360º). All the angles are in degrees with six decimal positions.

A row indicating that the list of blocks starts:

@@SOB@@

A number of lines, each one representing a CB, for example:

0   |288.213014799|18.188071449|146.093757802

1   |8.130439267 |38.485780519|78.149106404

…

A row indicating that the list of blocks ends:

@@EOB@@

A comment row (ignored by the S/W) with the brief indication of the meaning of the different columns for each source to be included in this output file:

# Name|Ra|Dec|Mag|Type|Priority|BlockId|PosId|Comment

These correspond to the Source Name, Right Ascension and Declination of the source, Magnitude, Type of source, Priority, Block Id, Positioner Id and a Comment field. All the angles are in degrees with six decimal positions.

A row indicating the list of sources starts.

@@SOS@@

A number of lines, each one representing a source, for example:

f:0s:1|288.201412467|18.177383634|22.08|SOURCE|0|||foo comment

f:0s:2|288.211016382|18.154690987|15.98|SOURCE|5|0|2|foo comment

…

Note that in this example the first source is an unassigned source (empty assigned block and positioner), while the second is an assigned one (in this case this source is assigned to the block 0 and positioner 2).

A row indicating the list of sources ends.

@@EOS@@

For each source, in each row, we can find 9 fields separated by vertical bars (|) corresponding to parameters of a given Sky Point (SP). The fields are the following ones:

- Name: Name of the source (catalog name, e.g. Mk348, NGC4151, etc.)

- Ra: Right Ascension in J2000.0. The format is degrees with six decimal positions. The S/W is also prepared to understand coordinates expressed as hh:mm:ss.ss

- Dec: Declination in J2000.0. The format is degrees with six decimal positions. The S/W is also prepared to understand coordinates expressed as +dd:mm:ss.ss

- Mag: Magnitude. In this release this field interprets that magnitudes are all in the same band and photometrical system. However, in the next releases we are evaluating the possibility of adding to this field an alphanumeric string that allows the S/W to identify the photometrical system and the filter. The implementation of this functionality is TBD. The Query Panel allows searching the sources in a magnitude range by setting the brightest and the faintest magnitude to be selected.

- Type: There are 4 types of SPs in the input preselected-sky-point list for the FMAT:

SOURCE: target of scientific interest. A scientific SP corresponds to a source in which the user is interested in assigning a RP in a Configuration Plan (CP) to successfully observe it.

REFERENCE: target to be used as reference. It could have scientific interest or not. Reference-source SP corresponds to a source (star) that can be used to validate the correct position of the telescope and PA. It is generally recommended that every observation (at least for long exposures) contains, at least, three reference-source SPs.

BLANK: sky background source. A blank SP is a point in a blank-sky region of the field. It is generally recommended one blank SP at least in every observation with the Fiber MOS. The blank SP observation will be used in the DFP/DRP and quick-look tools for sky subtraction.

UNKNOWN: unknown type point (type has not been previously identified).

- Priority: level of priority assigned to each source. The default highest and lowest priorities are “0” and “10” respectively. The software allows to modify these values by mean of the parameters megara.fmat.minimum_priority and megara.fmat.maximun_priority in the  fmat.props file.

- BlockId: Block Id. Will be empty for unassigned sources.

- PosId: Positioner Id. Will be empty for unassigned sources.

- Comment: Field for comment.

All the fields should be present but some of them can be empty by simply two consecutive vertical bars (||). Compulsory non-empty fields are RA and DEC. If a source is assigned, both the BlockId and the PosId must be present. Conversely if a source is not assigned both the BlockId and the PosId must be empty.

We give an example of one assignment file in an intermediate stage in which different CBs have been created, but the assignment of the RPs is not yet final. The file name is SampleMegaraAssignationFile.maf and can be found at the same level within the S/W distribution as where the FMAT jar file is located.

### Configuration Block file set from the FMAT: The FMPT input

The format of the output file is similar to the input file but with the following differences (see Figure 7).

- Each configuration block and associated sources are represented within a single file.

- Only one block is allowed between the @@SOB@@ and @@EOB@@ marks.

- There are three more column fields (12 in total): The X, Y position of the positioner and

whether the positioner is enabled/disabled.

Figure 7: FMAT. Output example ASCII file.

These fields are:

- X (mm): Positioner x co-ordinate (mm with six decimals) in the Fiber MOS co-ordinate system.

- Y (mm): Positioner y co-ordinate (mm with six decimals) in the Fiber MOS co-ordinate system.

- Enabled: Boolean field to indicate if the RP identified by the PosId is enabled or disabled.

The compulsory fields needed for the FMPT are the PosID, X(mm) and Y(mm), and also the Priority and Enabled columns information.

# INSTALLING / execution

The tool is distributed in a single Java jar format and it is executable on Linux distributions, Windows and Mac OS X.

Once it is downloaded it can be executed by simply double clicking within the windows manager or from the console by simply typing:

java –jar fmat-x.y.z.jar

Where x.y.z should be substituted by the particular version number of the released jar file.

## Software requirements

The minimum software requirements to install FMAT on your computer are the following:

- Any operative system that allows Java Runtime.

- The Java Runtime Environment 7.0 or higher.        (http://www.java.com/en/download/manual.jsp)

- Internet connection: Upon the first installation, the FMAT needs Internet connection to download the Hipparcos catalog. But then, the Internet connection is not needed.

## Local Directory structure

Upon running FMAT the first time, the tool generates a directory in the user home directory called .fmat. Inside this directory the following structure is found (see Figure 8):

.fmat/

├── catalogs

│   └── hipparcos.vot

├── data

├── plans

├── fmat.props

├── fmatFileChooser.dat

├── fmatLayout

├── fmatPositionerIds.dat

└── usermanual.pdf

- catalogs: This directory contains catalogs to be used by the tool. In this release only the Hipparcos catalog (hipparcos.vot) is defined.

- data: To store possible future temporary data structures (such as overlay images, etc.)

- plans: This is the directory in which the different CBs associated to a CP are saved (once all CBs in the CP are validated and the CP itself is generated). For each CP a sub-directory named as yymmdd_hhmmss is created when the CP is finally generated. For example, the sub-directory called 20150121_094551 shall include the CBs saved as a common CP the day 21st of January 2015 at 09:45:51 (see Figure 9 and Figure 10).

Figure 8: FMAT. Local directory structure created under the user’s home directory.

Figure 9: FMAT. Local sub-directory /.fmat/plans where CPs sub-directories are created.

Figure 10: FMAT. Local sub-directory with CBs associated to a given CP.

The tool also create in the .fmat directory the following files:

fmat.props, the fmatFileChooser.dat, the fmatLayout, the fmatPositionerIds.dat and the usermanual.pdf.

### The fmat.props configuration file

The file fmat.props contains the configurable parameters that are expected by the tool. These parameters are read by the tool upon starting it. The color properties are here omitted

megara.fmat.positioner.axis_radius=5.8025

megara.fmat.positioner.distance=20.1

megara.fmat.plate_scale=1.2133

megara.fmat.positioner.head.radius=2.35

megara.fmat.positioner.head.length=7.35

megara.fmat.positioner.head.margin=0.25

megara.fmat.positioner.lenslet_radius = 0.766

megara.fmat.lcb.sizex=9.96

megara.fmat.lcb.sizey=8.85

megara.fmat.default_frame=EQUATORIAL

megara.fmat.default_projection=Orthographic

megara.fmat.maximun_priority=0

megara.fmat.minimum_priority=10

megara.fmat.out_of_order_positioners=

megara.fmat.properties.update=true

megara.fmat.properties.url=http://pastebin.com/raw.php?i=vG3ysrZA

megara.fmat.strict=false

# Initial minimum memory in Megabytes

megara.fmat.min_memory=750

It contains configuration parameters for the coordinates, projection and colors of the different layout elements. The user could configure these colors and projections.

The following parameters:

megara.fmat.maximun_priority = 0

megara.fmat.minimum_priority = 10

allow defining the priority range. By default, this range has been configured between “0” (the highest priority) and “10” (the lowest priority).

The parameter:

megara.fmat.out_of_order_positioners = 31,70

It defines, separated by comas, the out-of order (disabled) RPs. In this example these are the 31st and 70th (those shown in red in Figure 6). In the initial configuration file coming with the release this field is empty and should be updated when a positioner happens to be out of order. These positioners will be red-painted and the tool will consider their exclusive patrol areas as inaccessible.

The following parameters:

megara.fmat.positioner.axis_radius = 5.8025

megara.fmat.positioner.distance = 20.1

megara.fmat.positioner.head.length = 7.35

megara.fmat.positioner.head.margin = 0.1

megara.fmat.positioner.head.radius = 2.35

define the basic geometry of the RPs and are (all in mm) the axes radii (each positioner is articulated on two rotations of the same radius), distance between neighbor positioners, positioner head length, positioner head margin and positioner head radius (see Figure 11).

Figure 11: Positioners geometry.

The following parameter:

megara.fmat.plate_scale=1.2133

defines the plate scale of the telescope.

The parameter:

megara.fmat.strict=true

will set whether the tool allows the user to generate incorrect configuration plans. If set to true the system will prevent to do so while if set to false, the tool will generate a warning message reporting the found inconsistencies and ask the user to confirm the generation.

### The remote properties service

The aforementioned properties are the default ones, however a remote properties service to be managed by GTC are needed. This service will allow GTC to update some properties that are prone to change in time as for example the disabled positioners.

For the time being, this functionality is implemented as a pastebin service managed by the user megara-fmat. The permanent url is http://pastebin.com/raw.php?i=vG3ysrZA

megara.fmat.properties.update=true

megara.fmat.properties.url=http://pastebin.com/raw.php?i=vG3ysrZA

If the property megara.fmat.properties.update is set to true and there is internet connectivity, the properties in this service will supersede any other defined by the user in his local fmat.props file. A backup copy of previously defined user properties will be stored in the same directory and the local user properties file will be also updated.

# USING THE FMAT

## Starting

To load the FMAT the user has to double-click in the jar file (fmat-2.3.1.jar for this release) in the window manager or starting it from the console as described in section 4. The only requirement for loading the tool is to have Java installed as described in section 4.

Upon staring, the FMAT interface appears as shown in Figure 12.

Figure 12: FMAT display when opening the tool.

## FMAT Graphic User Interface Description

The interface is composed by:

- The Main Menu (in the top-left part of the screen).

- The Panels

  - The Schedule panel (by default appears at the top of the screen). This window has no functionality in this release but it is prepared to display events as a function of time.

  - The Sky panel (background in black) where the sky view is displayed (in orthographic projection by default).

  - The Tree View panel (by default appears at the right).

  - The Query panel where some search functionality to filter the catalog is offered to the user.

  - The Sources panel (by default appears at the bottom-left) in which the user’s source list shall be displayed and will be automatically updated when filtering the data in the Query Window.

  - The Blocks panel (by default appears at the bottom-right).

The panels’ layout can be arranged as desired by means of the panel tabs operation: dock/undock, maximize, minimize and close. At any point the default layout can be restored by means of the File->Restore layout option. It can be also saved as described below.

### Main FMAT Menu

In this menu we find the following tabs:

- File

  - Load: This tab allows the user to select a file with his own catalog for MEGARA observations. The tool has the demo file SampleMegaraAssignationFile.maf. The file is selected with the appropriate navigation within each operative system.

  - Save: This tab allows the user to save a file so to store the current assignment work, in the stage in which the user decides to temporarily interrupt the assignment. This will be a new .maf file in which the astronomers will find their own catalog plus all the information (blocks, assigned positioners, etc.) resulted from their work with the FMAT. They will be able to start the FMAT from this file in a future FMAT session.

  - Generate Configuration Plan: This tab allows the user to generate a Configuration Plan (CP) composed by N Configuration Blocks (CBs). The FMAT will generate a directory called plans with a subdirectory labeled as yyyymmdd_hhmmss.

  - Print: This tab allows the user to print (on a printer previously configured for that computer) the content of the Sky panel.

  - Take sky panel screenshot: This tab allows the user to capture an image (.png) of the content of the Sky panel.

  - Save Layout: This tab allows the user to save the application layout.

  - Restore Layout: This tab allows the user to restore the saved layout (or the initial one – by default).

  - Clear: This tab allows the user to clear the load catalog for MEGARA observations and CBs.

  - Quit: This tab allows the user to exit the FMAT. Before executing this command the FMAT opens a pop-up asking if you really want to quit (and the user can answer yes – and quit – or not – and ignore this command).

- Edit

  - Undo: This tab allows the user to undo the last action about the current assignment work. The actions that can be undone are: move block, rotate block, add block, clone block, assign block, clear block, remove block, assign source and de-assign source.

  - Redo: This tab allows the user to redo the last action about the current assignment work. The actions that can be redone are: move block, rotate block, add block, clone block, assign block, clear block, remove block, assign source and de-assign source.

- View

This panel allows the user to configure the sky projection and the sky reference coordinates.

  - Projection

At the zoom level of the small MEGARA FOV all projections are virtually identical and equally valid. It is suggested to use the by-default “Orthographic” projection, although the following projections are also offered.

- Orthographic

- Gnomonic

- Mollweide

- Hammer

- Cylindrical

  - Frame

- Ecliptic

- Equatorial (J2000.0)

- Galactic

- Block

  - New Block at center: This tab allows the user to create a new MOS Configuration Block (CB) at the center of Sky View Panel. This will generate an empty configuration of the MEGARA MOS showing all the positioners and their numbers but without any source assigned to them.

  - New Block at: This tab allows the user to create a new MOS Configuration Block (CB) at specific sky position by filling central coordinates (RA and DEC) and PA (position angle). This will generate an empty configuration of the MEGARA MOS showing all the positioners and their numbers but without any source assigned to them.

  - Assign block: This tab allows the user to automatically assign sources to the selected block (see 5.6).

  - Clear block: This tab allows the user to automatically de-assign sources to the selected block (see 5.6).

- Tools

Check: This option checks whether the current configuration is feasible. In this release, the checks that are performed are:

- All the positioners are assigned to sources within the range of each positioner.

- All the positioners are not closer than the security radius defined in megara.fmat.positioner.head.margin

It should be noted that the collision detection (including the security radius) is performed taking into account the full geometry of the positioners head.

- Options

Tooltip: This option actives/deactivates the tooltip (the pop-ups that appear when you hover over any CB or source) in the sky panel.

- Help

  - Manual: This tab allows the user to open the User Manual.

Window Panels

All the panels include some indications as follows:

At top-left

The window label and three symbols: a curved arrow, an underscore and a cross. The functionality of these symbols is common to all windows and it is as follows:

- Clicking the curved arrow the user can undock the window and place it anywhere.

- Clicking the underscore the user can minimize the window size.

- Clicking the cross, the user can remove the window.

Figure 13: FMAT. Sky View Panel.

At top-right we can find the same symbols plus the symbol to maximize the window size.

All windows can be freely moved within the canvas.

To save the configuration of the application layout we can use the main menu with the tab “Save Layout” and always we can restore it with the main menu tab “Restore Layout” (see ).

### The Schedule panel

This window has no functionality in this release but it is prepared to display events as a function of time (by default appears at the top of the screen). It can be removed without side effects.

### The Sky panel

This panel is used to represent the sky in the selected projection (see Figure 13). The projection type by default can be configured in the fmat.props file. Otherwise, the tool starts with the default orthographic projection. The projection type can be changed in the tab View of the Main FMAT Menu at any moment.

In addition to the standard window labels, the Sky Panel shows the RA (α) and Dec (δ) coordinates in the J2000.0 equinox of the coordinates under the pointer when hovering within this panel, along with the FOV of the sky shown (this value represents the diagonal of the field shown in the Sky Panel in arcmin). Also, at the bottom-right there are some extra-help to move the window, up, down, left, right and to zoom (in-out). Also, we can zoom in with the mouse wheel towards us and zoom out with the mouse wheel rotating towards the monitor (as long as the mouse pointer is not placed at the very center of a Block).

### The Tree View panel

The Tree View panel (see Figure 12) includes the possibility of displaying catalogs together with the user’s catalog (the Hipparcos catalog has been included as an example) and also to paint different Lines (Grid, Ecliptic lines, Equatorial lines and constellations).

### The Query panel

This panel allows filtering the current sources by different criteria. The FMAT allows the user to filter by a range in priority, by a range in magnitude (see Figure 14), by characters in the field comment, allows selecting the focal-plane cover (left, right or removed) and also allows selecting assigned, unassigned and all sources. The source panel is refreshed automatically and both the Sky panel and the Source panel will show the targets resulting from this query only.

Figure 14: FMAT. Query Panel on top of the other windows.

The focal-plane cover query functionality deserves further discussion: by selecting the Left, Right and Removed options the user selects those sources at right, left or all respectively. Since several blocks may be present in the system, the query is performed upon the one that is selected in the blocks table panel (see Figure 15).

Figure 15: Right panel selected. Only those sources in the left half of the block are visible.

### The Sources panel

The Source Panel shows the information of interest on the user’s catalog (see Figure 14, bottom left). The user’s catalogue shall be displayed if this has been loaded properly and it is automatically updated when filtering the data in the Query Window. When clicking in a source row then the Sky Panel displays the source at the center of the field with a red pointer.

### The Blocks panel

The Blocks Panel shows the information of the Configuration blocks (see Figure 14, bottom right). The Configuration blocks list shall be displayed (if any block has been created or blocks were defined already in the input catalogue) and shall be also automatically updated when filtering the data in the Query Window. When clicking in a block row then the Sky Panel displays that block centered in the field (see Figure 16). The selection of sources pertaining to certain blocks can be performed by the associated check boxes.

The assigned sources appear colored in “yellow” circles and with a size proportional to their priority (bigger circles denotes higher priority objects).

By clicking in the check box of the block row (first column) you can also display or not in the Sky panel the block and the sources assigned to this block. This is very useful in the case in a region you have to many sources (see Figure 18).

Figure 16: Block “0” has been selected and shown in the Sky Panel.

Figure 17: Application Layout configured for the user. This configuration can be saved and restored.

Figure 18: This is an example where, in the left the Sky panel displays all the sources (assigned and unassigned) and all the blocks (in this case 9); and in the right the Sky panel displays all unassigned sources and one block with the sources assigned to this block.

## How to create a new block

You can create a new block by clicking in the tab Block of the Main Menu and select:

- New block at center: The block is created at the center of the Sky View Panel, which has been offset from the position of the region of interest to avoid overlapping (see Figure 19).

Figure 19: Creating a New Block at center from the Main Menu

- New block at ...: The block is created in a specific sky position by filling central coordinates (RA and DEC in degrees with three decimal positions) and PA (position angle in degrees with three decimal positions), see Figure 20.

Figure 20: Creating a New Block at a specific sky point from the Main Menu.

Figure 21: An empty MOS layout is created for the new block in the Sky Panel.

Figure 22: The new block layout (right) is created and appears in the Sky View Panel, which has been offset from the position of the region of interest to avoid overlapping. In this case Block “8” has been created. The layout appears in the Sky panel and the central coordinates and PA appear in the Blocks panel.

A new block is created “empty” without any source assigned to this block. When creating a new block a new MOS layout is created (see Figure 21). To avoid overlapping with the layout of another block, we recommend creating the block in an empty zone of the Sky View panel and then move it with the mouse to the region of interest (see Figure 22).

## How to remove a block

The blocks can be removed in the Sky panel. A block can be removed by right clicking on the green rectangle (which depicts the LCB) and selecting Remove block (see Figure 23 and Figure 25). The Blocks panel is then automatically refreshed.

The blocks can be removed in the Block panel by clicking the block row and then click the right mouse button (see  Figure 24 and Figure 25). The Sky panel is then automatically refreshed.

Figure 23: A block can be removed by clicking the green rectangle and then click the right mouse button and select Remove block.

Figure 24: A block can be removed by clicking the block row in the Block View Table and then click the right mouse button and select Remove block.

Figure 25: A pop-up reminds the user the consequence of removing the block.

## How to assign and de-assign sources to the RPs

You can assign/de-assign sources by clicking on the open circle that represents the source.  The circle changes to a filled yellow circle when the source is assigned to a RP.

Also the positioner header representation move from the center of the RP to the assigned source. This will allow the user to quickly identify the RPs to which the source has been assigned and eventually to see if a change in the center of the FOV or the PA can invalidate some of the assignments.

Figure 26: A pop-up with Source information is displayed when moving the mouse on a source on the MEGARA FOV.

Figure 27: A pop-up with Source information is displayed when moving the mouse on a source.

To remove an assignment just click again on the yellow circle (the source representation) and the circle will become open again and the positioner head will move to the center of the RP. The block and RP assignations will be automatically updated in all panels: Sky, Sources and Blocks.

The mouse can be positioned on a source and then a tooltip will arise with the information of the source (see Figure 26 and Figure 27). This tooltip can be deactivated as described before.

The center of the FOV can be moved by first clicking the green rectangle, to activate the movement, and then dragging the center of the FOV with the mouse. In order to change the PA, instead of moving the mouse, the user should move the wheel of the mouse. Both, the coordinate center and the PA are displayed on the bottom of the Sky View panel. If the CTRL key is simultaneously pressed, the rotation step is decreased to allow finer positioning.

When moving the FOV (or rotating the PA) some of the sources (that had been preliminary assigned to RPs) can move out of their patrol areas. When this happens, the positioner header representation of the RP becomes red. This helps the user to be aware of the consequence of these changes (see Figure 28, Figure 29 and Figure 30).

Figure 28: After clicking the green rectangle the user can change the FOV center and PA with the mouse. This can provoke that some sources that had been previously assigned go out of the patrol area and when this happens the corresponding positioner header representations change from green to red.

Figure 29: Example of the result of the movement of the FOV with the mouse or with the bottom right buttons of the Sky View panel. All assignments remain valid.

Figure 30: Detail of another movement provoked by the user. In this case some of the sources go out of the pre-assigned RPs.

## How to assign and de-assign sources automatically to a block

You can automatically assign sources to a block by clicking the green rectangle in the Sky panel and then click the right mouse button and select Assign (see Figure 23); or by clicking the block row in the Block panel and then click the right mouse button and select Assign (see Figure 24); or click on the green rectangle to select the block and then click in the tab Block of the Main Menu and select Assign block. See the result in Figure 31.

This automatic assignment assigns for each positioner the source (if any) that is attainable by the positioner. If several sources are attainable, then the procedure selects that of highest priority. If multiple sources are available with the same priority one of them is randomly selected.

It should be noted that this procedure can produce configurations in which some positioners collide with each other. These situations will be denoted by painting the affected positioners in red. It is then up to the user to resolve these conflicts.

The block and RP assignations are automatically updated in all panels: Sky, Sources and Blocks.

To remove the RP assignations to the block just click in the green rectangle in the Sky panel and then click the right mouse button and select Clear (see Figure 23); or click the block row in the Block panel and then click the right mouse button and select Clear (see Figure 24); or click on the green rectangle to select the block and then click in the tab Block of the Main Menu and select Clear block.

The block and RP assignations are automatically updated in all panels: Sky, Sources and Blocks.

Figure 31: At left we show the empty block while at right we show the block after the use of the automatic assignment feature.

## Choice between two possible RPs

When assigning individual positioners, sometimes the source can be accessed by two RPs (see Figure 32). In the example the source could be assigned to (and observed by) either RP #53 or RP #55.

Figure 32: Example of a source that can be assigned to two RPs.

The tool shows a pop-up from which the user can select the positioner to which the source should be assigned to (see Figure 33).

Figure 33: The FMAT assists the user to select between potential RPs to assign the sources.

## How to clone a block

You can clone a block by clicking on the green rectangle of the block you want to clone and then click the right mouse button and select Clone (see Figure 23).

Clone a block means to create a new block with exactly the same sky position (defined with the central coordinates RA and DEC and the position angle) that the cloned block. This new block is “empty”, i.e. there are no sources assigned to it.

## Save the blocks

Should the user decide to quit the tool, the last configuration of the Source catalog would be saved. This catalog can be used in future sessions until finishing the assignment.

The blocks can be saved by selecting “Save -> Save blocks” under the File tab in the Main Menu, as shown in Figure 34.

Figure 34: Save blocks from the Main Menu.

## Finishing the work: How to save a Configuration Plan

Once the user is satisfied with the created Configuration Blocks and the assignments to the sources done, a Configuration Plan (CP) can be generated. The CP will be composed by a number of Configuration Blocks with all the assignment information.

First, the CP can be validated by selecting "Check" under the Tools tab in the Main Menu. Once the CP is validated the user should make use of the "Generate configuration plan" option in the File tab of the Main Menu as shown in Figure 35.

Figure 35: Generation of a Configuration Plan (CP) from the Main Menu.

A validation process (the same validation as the one performed in Tools=>Check) is executed as a pre-requisite to the generation of the configuration plan. Should this validation not be successful, a warning message would be presented to the user and the generation is not performed (see Figure 36).

The property megara.fmat.strict controls whether incorrect plans can be generated. Should there are still issues with blocks at configuration plan generation, a warning message is presented to the user. If the property is set to true the configuration plan will not be generated asking the user for correction measures, while if set to false, the tool will give the choice to the user to abort or to generate the incorrect configuration plan, see Figure 36.

Figure 36: Trying to generate a configuration plan with issues when the megara.fmat.strict is set to false (left panel) and true (right panel).

The FMAT will create a directory under the user's home directory named .fmat/plans/ where the entire Configuration Plan would be saved inside a sub-directory named  .fmat/plans/yymmdd_hhmmss. The name of the directory refers to the date and time of creation of the CP. FMAT will then save the different text/ASCII files for each of the different CBs (see Figure 37,

Figure 38 and Figure 39).

Figure 37: The FMAT indicates the directory where the CBs of the CP have been created.

Figure 38: Directory where the CP is saved (a CP is a set of CBs)

.

Figure 39: Directory of a CP showing the different text files corresponding to the different CBs.

By default inside the user’s home in .fmat/

The definition of the plate scale assumes the gnomonic or tangential projection on the focal plane. It is TBD whether additional refinements, and therefore parameters will be needed to take into account the departure from this model (geometrical distortion).

http://pastebin.com/

In some modern trackpads the zoom gesture corresponds with up/down with two fingers.